SPIE Optics + Photonics Abstract Submission Guidelines

- https://spie.org/conferences-and-exhibitions/optics-and-photonics/presenters/abstract-submission-guidelines

Presentation title

BRDF measurements in support of stray light design for space telescope programs at UA

Author(s) information

- Jacob P. Krell (Wyant)

- Heejoo Choi (Wyant, Steward)

- Maggie Y. Kautz (Steward)

- Cameron C. Ard (Steward)

- Pierre Nicholas (Wyant)

- Matthew E. Mavko (Wyant)

- Nikolaus Anderson (Wyant)

- Daewook Kim (Wyant, Steward)

- Ewan S. Douglas (Steward)

Speaker biography (1000-character max including spaces)

Jacob Krell is a graduate student at the University of Arizona Wyant College of Optical Sciences. He is a member of the LOFT research group, advised by Prof. Daewook Kim and Prof. Heejoo Choi. His career interest is primarily in the design and integration of both ground- and space-based telescopes for astronomy and space exploration.

Abstract for technical review (200-300 words; text only)

Measurements of light scatter from paints, coatings, and optomechanical materials are often unavailable for the specific materials used in an optical instrument. As a result, stray-light simulations frequently rely on simplified assumptions or surrogate datasets. Direct measurement of scatter from the materials of interest can improve the fidelity of these simulations.

The measurements presented here were performed in support of stray-light design analysis for the Lazuli Space Observatory. Bidirectional reflectance distribution function (BRDF) measurements were acquired using a J&C RT-300S scatterometer at the Wyant College of Optical Sciences, together with an internally developed calibration and data-processing routine. The emphasis of this work is practical: to obtain accurate scatter data suitable for optical modeling using the available instrument.

Isotropic BRDFs are reported for Anoplate AnoBlack NiTE on steel and INVAR, and for Anoplate AnoBlack EC1 and EC2 on aluminum. The NiTE on INVAR measurements demonstrate the value of this in-house capability, as these samples correspond to custom off-axis parabolic (OAP) mirror mounts manufactured for the ExtraSolar Coronagraph (ESC) instrument. In addition, measurements are presented for an anisotropic and highly diffractive SONY IMX455 CMOS detector surface intended for use in the Widefield Context Camera (WCC), where ghost reflections are of concern.

We describe the measurement geometry, data handling, and post-processing steps required to generate BRDF datasets compatible with Zemax and FRED. Internal consistency checks and comparisons with manufacturer data are used to verify the results. The resulting datasets are released for use in stray-light simulation studies, with an emphasis on transparency and practical applicability.

Summary of abstract for display in the program (50-150 words; text only)

This work presents Bidirectional Reflectance Distribution Function (BRDF) measurements performed in support of stray light design for space telescope programs at the University of Arizona, including the Lazuli Space Observatory. Primary measurements were obtained using a J&C RT-300S scatterometer and converted to BRDF datasets using an internally developed calibration and data processing routine to improve measurement accuracy. Additional measurements from the Remote Sensing Group’s Blacklab extend BRDF characterization across Lazuli’s full science spectral band. Samples include isotropic black coatings and an anisotropic CMOS detector surface relevant to ghost reflections. The resulting BRDF datasets are released for use in Zemax and FRED stray light simulations.

Keywords used in search for your paper (optional)

BRDF, BSDF, scatterometry, stray light, reflectance, optical coatings, coronagraph, Lazuli Space Observatory